## Lab 9.6.2: Challenge EIGRP Configuration Lab

## Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Branch1 | Fa0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 |  |  | N/A |
| HQ | Fa0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 |  |  | N/A |
|  | Lo1 |  |  | N/A |
| Branch2 | Fa0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 |  |  | N/A |
| PC1 | NIC |  |  |  |
| PC2 | NIC |  |  |  |
| PC3 | NIC |  |  |  |

### Learning Objectives

Upon completion of this lab, you will be able to:

- Create an efficient VLSM design given requirements.

- Assign appropriate addresses to interfaces and document.

- Cable a network according to the Topology Diagram.

- Erase the startup configuration and reload a router to the default state.

- Configure routers including EIGRP.

- Configure a static default route.

- Verify EIGRP operation.

- Test and verify full connectivity.

### Scenario

In this lab activity, you will be given a network address that must be subnetted using VLSM to complete the addressing of the network shown in the Topology Diagram. A combination of EIGRP routing and static routing will be required so that hosts on networks that are not directly connected will be able to communicate with each other. EIGRP must be configured so that all IP traffic takes the shortest path to the destination address. Use autonomous system 1 for the EIGRP configuration.

### Task 1: Subnet the Address Space.

#### Step 1: Examine the network requirements.

The addressing for the network has the following requirements:

- The 172.16.0.0/16 network must be subnetted to provide addresses for the three LANs.

  - The HQ LAN will require 500 addresses.

  - The BRANCH1 LAN will require 200 addresses.

  - The Branch 2 LAN will require 100 addresses.

- The loopback address representing the link between the HQ router and the ISP will use the 209.165.200.224/30 network.

- The 192.168.1.16/28 address space must be subnetted to obtain the addresses for the links between the three routers.

#### Step 2: Consider the following questions when creating your network design:

How many subnets need to be created from the 172.16.0.0/16 network? __3____
 
How many total IP addresses are required from the 172.16.0.0/16 network? __800___ 

What subnet mask will be used for the HQ LAN subnet? ___/23 (255.255.254.0)_

What is the maximum number of host addresses that could be used on this subnet? _510____ 

What subnet mask will be used for the BRANCH1 LAN subnet? __/24 (255.255.255.0)_ 

What is the maximum number of host addresses that could be used on this subnet? __254__ 

What subnet mask will be used for the BRANCH2 LAN subnet? /25 (255.255.255.128) 

What is the maximum number of host addresses that could be used on this subnet? __126__

What subnet mask will be used for the links between the three routers?

/30 (255.255.255.252) 

What is the maximum number of host addresses that could be used on each of these subnets? _2___

#### Step 3: Assign subnetwork addresses to the Topology Diagram.

- Assign subnet 0 of the 172.16.0.0/16 network to the HQ LAN subnet. 
What is the network address of this subnet? _________172.16.0.0 /23 _______

- Assign subnet 1 of the 172.16.0.0/16 network to the BRANCH1 LAN subnet. 
What is the network address of this subnet? _________172.16.2.0 /24______

- Assign subnet 2 of the 172.16.0.0/16 network to the BRANCH2 LAN subnet. 
What is the network address of this subnet? _________172.16.3.0 /25_______

- Assign subnet 0 of the 192.168.1.16/28 network to the link between the HQ and BRANCH1 routers. 
What is the network address of this subnet? ______192.168.1.16 /30_________

- Assign subnet 1 of the 192.168.1.16/28 network to the link between the HQ and BRANCH2 routers. 
What is the network address of this subnet? _____192.168.1.20 /30________

- Assign subnet 2 of the 192.168.1.16/28 network to the link between the BRANCH1 and BRANCH2 routers.
What is the network address of this subnet? ________192.168.1.24 /30_______

### Task 2: Determine Interface Addresses.

#### Step 1: Assign appropriate addresses to the device interfaces.

- Assign the first valid host address of the 209.165.200.224/30 network to the Loopback interface on the HQ router.

- Assign the first valid IP address of the HQ LAN network to the LAN interface of the HQ router.

- Assign the last valid IP address of the HQ LAN network to PC2.

- Assign the first valid IP address of the BRANCH1 LAN network to the LAN interface of the BRANCH1 router.

- Assign the last valid IP address of the BRANCH1 LAN network to PC1.

- Assign the first valid IP address of the BRANCH2 LAN network to the LAN interface of the BRANCH2 router.

- Assign the last valid IP address of the BRANCH2 LAN network to PC3.

- Assign the first valid IP address of the HQ to BRANCH1 link network to the Serial 0/0/0 interface of the HQ router.

- Assign the last valid IP address of the HQ to BRANCH1 link network to the Serial0/0/0 interface of the Branch router.

- Assign the first valid IP address of the HQ to BRANCH2 link network to the Serial 0/0/1 interface of the HQ router.

- Assign the last valid IP address of the HQ to BRANCH2 link network to the Serial0/0/1 interface of the Branch router.

- Assign the first valid IP address of the BRANCH1 to BRANCH2 link network to the Serial 0/0/1 interface of the BRANCH1 router.

- Assign the last valid IP address of the BRANCH1 to BRANCH2 link network to the Serial0/0/0 interface of the BRANCH2 router.

#### Step 2: Document the addresses to be used in the table provided under the Topology Diagram.

### Task 3: Prepare the Network.

#### Step 1 Cable a network that is similar to the one in the Topology Diagram.

You can use any current router in your lab as long as it has the required interfaces shown in the topology.

#### Step 2 Clear any existing configurations on the routers.

### Task 4: Perform Basic Router Configurations.

Perform basic configuration of the BRANCH1, BRANCH2, HQ, and ISP routers according to the following guidelines:

- Configure the router hostname.

- Disable DNS lookup.

- Configure an EXEC mode password.

- Configure a message-of-the-day banner.

- Configure a password for console connections.

- Configure a password for VTY connections.

- Synchronize unsolicited messages and debug output with solicited output and prompts for the console and virtual terminal lines.

- Configure an EXEC timeout of 15 minutes.

### Task 5: Configure and Activate Serial and Ethernet Addresses.

#### Step 1: Configure the interfaces on the HQ, BRANCH1, and BRANCH2 routers.

Configure the interfaces on the HQ, BRANCH1, and BRANCH2 routers with the IP addresses from the table provided under the Topology Diagram. When you have finished, be sure to save the running configuration to the NVRAM of the router.

#### Step 2: Configure the Ethernet interfaces.

Configure the Ethernet interfaces of PC1, PC2, and PC3 with the IP addresses from the Addressing Table provided under the Topology Diagram.

### Task 6: Verify Connectivity to Next-Hop Device.

You should not have connectivity between end devices yet. However, you can test connectivity between two routers and between an end device and its default gateway.

#### Step 1: Verify connectivity of routers.

Verify that the HQ, BRANCH1, and BRANCH2 routers can ping each of the neighboring routers across the WAN links.

#### Step 2: Verify connectivity of PCs.

Verify that PC1, PC2, and PC3 can ping their respective default gateways.

### Task 7: Configure EIGRP Routing on the BRANCH1 Router.

Consider the networks that need to be included in the EIGRP updates that are sent out by the BRANCH1 router.
What directly connected networks are present in the BRANCH1 routing table?
______172.16.2.0, 192.168.1.16, and 192.168.1.24__________

Will these networks need to have the subnet mask information included in the network statements? ___Yes___ 

What commands are required to enable EGIRP and include the connected networks in the routing updates? 
router eigrp 1

network 172.16.2.0 0.0.0.255

network 192.168.1.16 0.0.0.3

network 192.168.1.24 0.0.0.3

What command is required to enable EGIRP to include the VLSM information instead of summarizing routes at the classful boundary? no auto-summary
Are there any router interfaces that do not need to have EIGRP updates sent out? ___Yes___ 

What command is used to disable EIGRP updates on these interfaces?
________passive-interface fa0/0___________

### Task 8: Configure EIGRP and Static Routing on the HQ Router.

Consider the type of static routing that is needed on HQ.
A static default route will need to be configured to send all packets with destination addresses that are not in the routing table to the loopback address representing the link between the HQ router and the ISP. What command is needed to accomplish this? 
____ip route 0.0.0.0 0.0.0.0 loopback 1_____ 
What directly connected networks are present in the HQ routing table? 
__172.16.0.0, 192.168.1.16, 192.168.1.20, and 209.165.200.224____

Will the networks of the HQ LAN and the links between the BRANCH1 and BRANCH2 routers need to have the subnet mask information included in the network statements? ___Yes____ 

What commands are required to enable EGIRP and include the appropriate networks in the routing updates? 
router eigrp 1

network 172.16.0.0 0.0.1.255

network 192.168.1.16 0.0.0.3

network 192.168.1.20 0.0.0.3

redistribute static
What command is required to enable EGIRP to include the VLSM information instead of summarizing routes at the classful boundary? 
no auto-summary

Are there any router interfaces that do not need to have EIGRP updates sent out? ___Yes____ 

What command is used to disable EIGRP updates on this interface?
________passive-interface fa0/0___________

### Task 9: Configure EIGRP Routing on the BRANCH2 Router.

Consider the networks that need to be included in the EIGRP updates that are sent out by the BRANCH2 router.

What directly connected networks are present in the BRANCH2 routing table? 
_____172.16.3.0, 192.168.1.20, and 192.168.1.24______ 

Will these networks need to have the subnet mask information included in the network statements? ___Yes____ 

What commands are required to enable EGIRP and include the connected networks in the routing updates? 
router eigrp 1

network 172.16.3.0 0.0.0.127

network 192.168.1.20 0.0.0.3

network 192.168.1.24 0.0.0.3

What command is required to enable EGIRP to include the VLSM information instead of summarizing routes at the classful boundary? 
_________no auto-summary___________ 

Are there any router interfaces that do not need to have EIGRP updates sent out? __Yes____ 

What command is used to disable EIGRP updates on these interfaces?
_________passive-interface fa0/0_______

### Task 10: Verify the Configurations.

Answer the following questions to verify that the network is operating as expected:
From PC1, is it possible to ping PC2? ___Yes_____ 

From PC1, is it possible to ping the PC3? __Yes_____ 

The answer to the above questions should be yes. If any of the above pings failed, check your physical connections and configurations. Refer to your basic troubleshooting techniques used in the Chapter 1 labs. 
What EIGRP routes are present in the routing table of the BRANCH1 router? 
172.16.0.0/23, 172.16.3.0/25, 192.168.1.20/30, and the default route 0.0.0.0/0

What EIGRP routes are present in the routing table of the HQ router? 
172.16.2.0/24, 172.16.3.0/25, and 192.168.1.24/30.

What is the gateway of last resort in the routing table of the HQ router? 
0.0.0.0 to network 0.0.0.0

What EIGRP routes are present in the routing table of the BRANCH2 router? 
172.16.0.0/23, 172.16.2.0/24, 192.168.1.16/30, and the default route 0.0.0.0/0

### Task 11: Reflection

Why is it necessary to use disable automatic summarization with this network design?

Because the 172.16.0.0 Class B network space is separated by the 192.168.1.0 network links. If automatic summarization is left on, the routers will summarize their specific LAN subnets up to the classful boundary (172.16.0.0/16) and advertise that over the WAN links. This creates a discontiguous network issue where multiple routers claim to be the optimal path to the exact same /16 network, leading to suboptimal routing, load-balancing errors, and blackholed traffic.

(Bởi vì không gian mạng Class B 172.16.0.0 bị chia tách bởi các liên kết mạng 192.168.1.0. Nếu tính năng tự động tóm tắt được để ở chế độ bật, các router sẽ tự động gom (tóm tắt) các subnet LAN cụ thể của chúng về địa chỉ mạng mặc định theo lớp (classful boundary là 172.16.0.0/16) và quảng bá thông tin đó qua các liên kết WAN. Điều này tạo ra vấn đề mạng không liên tục (discontiguous network), trong đó nhiều router cùng nhận mình là đường đi tối ưu nhất đến chính xác cùng một mạng /16, dẫn đến tình trạng định tuyến không tối ưu, lỗi cân bằng tải và hiện tượng rớt/mất gói tin (blackholed traffic).)

If the routes in the routing table are summarized at the classful network boundary 17.16.0.0, the paths between the three routers will all have an equal cost and packets may not be sent using the route with the least hops.