1.3.2: Directly Connected Routes

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Fa0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.2.1 | 255.255.255.0 | N/A |
| R2 | Fa0/0 | 192.168.3.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 192.168.2.2 | 255.255.255.0 | N/A |
| PC1 | NIC | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 | NIC | 192.168.3.10 | 255.255.255.0 | 192.168.3.1 |

Introduction

This activity focuses on the routing table and how it is built. A router builds routing tables by first adding the networks for the IP addresses configured on each of the active interfaces for the router. (These networks are the directly connected networks for the router.) The focus of this activity is two routers, R1 and R2, and the networks supported through the configuration of the router interfaces. Initially, all interfaces have been configured with correct addressing but the interfaces are shutdown.

Learning Objectives:

- Describe the function of the routing table.

- Describe how a router manages directly connected routes in the routing table.

Task 1: Enable the Interfaces on Each Router.

All of the interfaces on the routers have been shutdown. With the interfaces down, no routing table exists for each router. Follow the steps below to view the addition of routes to the routing table:

Step 1 – Enable the FastEthernet interface on R1.

- Click R1 in the logical workspace.

- Select the CLI tab.

- From the CLI (command line interface) type the following commands:
Password:cisco
R1>enable
Password: class
R1#show ip route
(Note: The routing table is completely empty of any routes.)

- Activate the interface FastEthernet:
R1#configure terminal
R1(config)#interface FastEthernet 0/0
R1(config-if)#no shutdown
R1(config-if)#end

Step 2 – Examine the routing table on R1.

- From the CLI type the following commands:
R1#show ip route

- Are there any routes in the routing table? If so, list the route(s) below:
Yes

192.168.1.0/24 is directly connected, FastEthernet0/0

- If there are no routes in the routing table, retrace your steps and troubleshoot the problem.

Step 3 – Enable the Serial interface on R1.

- Click R1 in the logical workspace.

- Select the CLI tab.

- From the CLI type the following commands:
R1#configure terminal
R1(config)#interface Serial 0/0/0
R1(config-if)#no shutdown
R1(config-if)#end

Step 4 – Repeat steps 1 through 3 on R2.

- How many routes are in the routing table for R1? 2 routes

192.168.1.0/24 is directly connected, FastEthernet0/0

192.168.2.0/24 is directly connected, Serial0/0/0

- How many routes are in the routing table for R2? 2 routes

192.168.2.0/24 is directly connected, Serial0/0/0

192.168.3.0/24 is directly connected, FastEthernet0/0

- Now shut down the FastEthernet interface on R1.

- How many routes are shown in the routing table for R1 now?
1 route (Only the Serial network 192.168.2.0 remains. The LAN network disappears because the interface is no longer active)

Chỉ còn mạng 192.168.2.0 trên cổng Serial. Mạng 192.168.1.0 sẽ biến mất ngay lập tức vì cổng kết nối nó đã bị tắt

- Did the number of routes in R2 change? Explain:

No

R2 only knows about its own Directly Connected networks. It has no information about R1's LAN (192.168.1.0) yet. Therefore, shutting down a port on R1 does not change the physical status of R2’s interfaces or its internal routing table.

(Vì R2 chỉ biết các mạng kết nối trực tiếp với nó. Việc cổng Fa0/0 của R1 bị tắt không ảnh hưởng trực tiếp đến các cổng vật lý đang "Up" của R2. R2 vẫn thấy mạng 192.168.2.0 (nối với R1) và mạng 192.168.3.0 (nối với PC2) bình thường.)

- Enter global configuration and use the no shutdown command to bring the FastEthernet 0/0 interface back up.

Task 2: Verify connectivity using ping

- PC1 should be able to ping each interface on R1.

- PC2 should be able to ping each interface on R2.

Can the PCs successfully ping each other? Explain.
No

Even though the interfaces are "Up," the routers only have Directly Connected routes.

- R1 knows how to reach 192.168.1.0 and 192.168.2.0, but it has no route to reach PC2's network (192.168.3.0).

- R2 knows how to reach 192.168.3.0 and 192.168.2.0, but it has no route to reach PC1's network (192.168.1.0).

- For a ping to work, both routers need Static Routes or a Dynamic Routing Protocol (like RIP or OSPF) to tell them how to reach the distant "Remote Networks."

(Mặc dù các cổng đã "Up" và có các route trực tiếp (Directly Connected), nhưng R1 không biết đường đến mạng của PC2 (192.168.3.0).

R2 không biết đường đến mạng của PC1 (192.168.1.0).

Để hai PC ping được nhau, bạn cần cấu hình thêm Static Route (Định tuyến tĩnh) hoặc dùng các giao thức định tuyến động (như RIP, OSPF) để các Router "trao đổi" thông tin về các mạng ở xa với nhau.)

(2312678 – Nguyễn Lê Bảo Long – CTK47MMT)