Lab 2.8.2: Challenge Static Route Configuration

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Branch | Fa0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
| HQ | Fa0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 | 209.165.201.2 | 255.255.255.252 | N/A |
| ISP | Fa0/0 | 209.165.200.225 | 255.255.255.224 | N/A |
|  | S/0/0/0 | 209.165.201.1 | 255.255.255.252 | N/A |
| PC1 | NIC |  |  |  |
| PC2 | NIC |  |  |  |
| Web Server | NIC | 209.165.200.254 | 255.255.255.224 | 209.165.200.225 |

Scenario

In this lab activity, you will be given a network address, and view the subnets to complete the addressing of the network shown in the Topology Diagram. The Internet Service Provider has provided the addressing for the LAN connected to the ISP router and the link between the HQ and ISP routers. As there will be no dynamic Routing, static routes will need to be configured so that hosts on networks that are not directly connected will be able to communicate with each other.

Task 1: Subnet the Address Space

Step 1 Examine the network requirements.

The addressing for the LAN connected to the ISP router and the link between the HQ and ISP routers has already been completed. You have been given the 192.168.2.0/24 address space to complete the network design. Subnet this network to provide enough IP addresses to support 60 hosts.

Step 2 Consider the following questions when creating your network design

How many subnets need to be created from the 192.168.2.0/24 network? 4

What are the subnetwork addresses of the subnets? 

What is the subnet mask for these networks in dotted decimal format? 255.255.255.192

What is the subnet mask for the network in slash format? /26

How many usable hosts are there per subnet? 62

Subnet 0: _______ 192.168.2.0__________

Subnet 1: _______192.168.2.64__________

Subnet 2: ______ 192.168.2.128__________

Subnet 3: ______ 192.168.2.192__________

Step 3 Assign subnetwork addresses to the Topology Diagram.

· Assign subnet 1 to the LAN attached to HQ.
· Assign subnet 2 to the WAN link between HQ and Branch.
· Assign subnet 3 to the LAN attached to Branch.
· Subnet 0 will be available for future expansion.

Task 2: Determine Interface Addresses

Assign appropriate addresses to the device interfaces.

- Assign the first valid host address in subnet 1 to the LAN interface on HQ.

- Assign the last valid host address in subnet 1 to PC2.

- Assign the first valid host address in subnet 2 to the WAN interface on Branch.

- Assign the second valid host address in subnet 2 to the WAN interface on HQ.

- Assign the first valid host address in subnet 3 to the LAN interface of Branch.

- Assign the last valid host address in subnet 3 to PC1.

Task 3: Prepare the Network

Step 1 Ensure that the network is cabled the same as the one in the Topology Diagram.

Step 2 Clear any existing configurations on the routers.

Task 4: Basic Router Configurations

Perform basic configuration of the Branch, HQ, and ISP routers according to the following guidelines:

- Configure the router hostname

- Disable DNS lookup

- Configure an EXEC mode password

- Configure a message-of-the-day banner

- Configure a password for console connections

- Configure a password for VTY connections

Task 5: Configure and Activate Serial and Ethernet Addresses

Step 1 Configure the interfaces on the Branch, HQ, and ISP routers with the IP addresses from the table provided under the Topology Diagram. When you have finished, be sure to save the running configuration to the NVRAM of the router.

Step 2 Configure the Ethernet interfaces of PC1, PC2, and the Web Server with the IP addresses from the table provided under the Topology Diagram.

Task 6: Verify Connectivity to Next Hop Device

You should NOT have connectivity between end devices yet. However, you can test connectivity between two routers or between and end device an its default gateway.

Step 1 Verify that Branch can ping across the WAN link to HQ and that HQ can ping across the WAN link it shares with ISP.

Step 2 Verify that PC1, PC2, and the Web Server can ping their respective default gateway.

Task 7: Configure Static Routing on Branch

Step 1 Consider the type of static routing that is needed on Branch.

What networks are present in the Branch routing table? List the networks with slash notation.

- 192.168.2.128/26

- 192.168.2.192/26

What networks are missing from the Branch routing table? List the networks with slash notation.

- 192.168.2.64/26

- 209.165.201.0/30

- 209.165.200.224/27

Can one summary route that includes all of the missing networks be created?____Yes_____

How many WAN routes are available to traffic leaving the LAN connected to Branch? 1

Step 2 Because Branch is a stub router, we should configure Branch with a default static route point to HQ. Record the command below to configure a default static route using the appropriate exit interface.

_________________ ip route 0.0.0.0 0.0.0.0 Serial0/0/0______________________________

Step 3 View the routing table of Branch to verify the new static route entry. You should a “Gateway of Last Resort” set on Branch.

Without testing it first, do you think PC1 can now successfully ping PC2? _No___

Why or why not?

The HQ router does not yet have a return route configured to send reply packets back to the Branch LAN

(Router HQ vẫn chưa được cấu hình tuyến đường trả về để gửi các gói tin phản hồi lại mạng LAN của Branch).

Task 8: Configure Static Routing on HQ

Step 1 Consider the type of static routing that is needed on HQ.

What networks are present in the HQ routing table? List the networks with slash notation.

- 192.168.2.64/26

- 192.168.2.128/26

- 209.165.201.0/30

What networks are missing from the HQ routing table? List the networks with slash notation.

- 192.168.2.192/26

- 209.165.200.224/27

Can one summary route that includes all of the missing networks be created? ____No_____

HQ is in a unique position as the hub router in this hub-and-spoke topology. Traffic from the Branch LAN destined for the Internet must pass through HQ. HQ must be able forward any traffic it does not have a route for to the ISP. What kind of route do you need to configure on HQ to solve this problem?

__________ A default static route (Tuyến đường tĩnh mặc định)_______________

HQ is also the intermediary for any traffic from the Internet destined for the Branch LAN. Therefore, HQ must be able to route to that LAN. What kind of route you need to configure on HQ to solve this problem?

_____________ A standard static route (Tuyến đường tĩnh tiêu chuẩn).__________

Step 2-Configure HQ a static route to the Branch LAN using the Serial 0/0/0 interface of HQ as the exit interface. Record the command you used.

_____ ip route 192.168.2.192 255.255.255.192 Serial0/0/0____________________________

Step 3-Configure the HQ router with a default static route pointing to ISP using the “next-hop” IP address. Record the command you used.

____ ip route 0.0.0.0 0.0.0.0 209.165.201.1___________________________________

Step 4-View the routing table of R2 to verify the new static route entries.

Step 5- Without testing it first, do you think PC1 can now successfully ping PC2?

_Yes_ Why or why not?

Because Branch has a default route to send the ping request to HQ, and HQ now has a specific static route to send the reply back to the Branch LAN.

(Bởi vì Branch đã có định tuyến mặc định gửi yêu cầu đến HQ, và HQ hiện đã có tuyến đường tĩnh cụ thể để gửi phản hồi về lại mạng LAN Branch)

Step 6-Without testing it first, do you think PC1 or PC2 can now successfully ping the Web Server?

__No___ Why or why not?

Although the ping requests can reach the Web Server, the ISP router does not have a route configured to send the replies back to the 192.168.2.0/24 subnet.

(Mặc dù gói tin có thể tới được Web Server, nhưng router ISP chưa có tuyến đường để trả gói tin về cho dải mạng 192.168.2.0/24).

Task 9: Configure Static Routing on ISP

Note: In a real-world implementation of this topology, you would not be configuring the ISP router. However, your service provider is an active partner in solving your connectivity needs. Service provider administrators are human, too, and make mistakes. Therefore, it is important that you understand the type of errors an ISP could make that would cause your networks to lose connectivity.

Step 1-Consider the type of static routing that is needed on ISP

What networks are present in the ISP routing table? List the networks with slash notation.

- 209.165.200.224/27

- 209.165.201.0/30

What networks are missing from the ISP routing table? List the networks with slash notation.

- 192.168.2.64/26

- 192.168.2.128/26

- 192.168.2.192/26

Can one summary route that includes all of the missing networks be created? ____Yes___

Step 2-Using the “next-hop” IP address, configure ISP with a summary static route that includes all of the subnets that are missing from the routing table. Record the command you used.

_______ ip route 192.168.2.0 255.255.255.0 209.165.201.2___________________________

Step 3-View the routing table of ISP to verify the new static route entry.

Task 10: Verify the Configurations

Answer the following questions to verify that the network is operating as expected.
From PC2, is it possible to ping PC1? ___Yes____

From PC2, is it possible to ping the Web Server? ____Yes___

From PC1, is it possible to ping the Web Server? ____Yes____

The answer to the above questions should be ‘yes’. If any of the above pings failed, check your physical connections and configurations. Refer to your basic troubleshooting techniques used in the [Chapter 1] labs.

What routes are present in the routing table of Branch?

C 192.168.2.128/26 is directly connected, Serial0/0/0

C 192.168.2.192/26 is directly connected, FastEthernet0/0

S* 0.0.0.0/0 is directly connected, Serial0/0/0

- What routes are present in the routing table of HQ?

C 192.168.2.64/26 is directly connected, FastEthernet0/0

C 192.168.2.128/26 is directly connected, Serial0/0/0

S 192.168.2.192/26 [1/0] via Serial0/0/0

C 209.165.201.0/30 is directly connected, Serial0/0/1

S* 0.0.0.0/0 [1/0] via 209.165.201.1

What routes are present in the routing table of ISP?

C 209.165.200.224/27 is directly connected, FastEthernet0/0

C 209.165.201.0/30 is directly connected, Serial0/0/0

S 192.168.2.0/24 [1/0] via 209.165.201.2

Task 11: Reflection

If a default static route was not configured on Branch, how many individual static routes would be needed to for hosts on the Branch LAN to communicate with all of the networks in the Topology Diagram? ______3_____
If a summary static route was not configured on ISP, how many individual static routes would be needed to for hosts on the ISP LAN to communicate with all of the networks in the Topology Diagram? ________3___