Lab 2.8.3: Troubleshooting Static Routes

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Branch | Fa0/0 | 172.20.1.129 | 255.255.255.128 | N/A |
|  | S0/0/0 | 172.20.1.1 | 255.255.255.128 | N/A |
| HQ | Fa0/0 | 172.20.0.129 | 255.255.255.128 | N/A |
|  | S0/0/0 | 172.20.1.2 | 255.255.255.128 | N/A |
|  | S0/0/1 | 192.168.38.254 | 255.255.255.252 | N/A |
| ISP | FA0/0 | 192.168.39.65 | 255.255.255.192 | N/A |
|  | S0/0/1 | 192.168.38.253 | 255.255.255.252 | N/A |
| PC1 | NIC | 172.20.1.135 | 255.255.255.128 | 172.20.1.129 |
| PC2 | NIC | 172.20.0.135 | 255.255.255.128 | 172.20.0.129 |
| Web Server | NIC | 192.168.39.70 | 255.255.255.192 | 192.168.39.65 |

Scenario

In this lab, you will begin by loading configuration scripts on each of the routers. These scripts contain errors that will prevent end to end communication across the network. You will need to troubleshoot each router to determine the configuration errors, and then use the appropriate commands to correct the configurations. When you have corrected all of the configuration errors, all of the hosts on the network should be able to communicate with each other.

Task 1: Cable, Erase, and Reload the Routers

Step 1 Cable a network that is similar to the one in the Topology Diagram.

On all routers the Lan interface will be Fa0/0
Connection from Branch to HQ is S0/0/0 to S0/0/0
Connection from HQ to ISP is S0/0/1 to S0/0/1

Step 2 Clear the configuration on each of routers using the erase startup-config command and then reload the routers. Answer ‘no’ if asked to save changes. (The enable password is class.)

Step 3 Configure the hosts with the addresses in the table.

Task 2: Load Routers with the Supplied Scripts

Step 1 Load the following script onto the Branch router. Make sure you are in global configuration mode prior to pasting the script to the router.

hostname Branch
!
!
no ip domain-lookup
!
interface FastEthernet0/0
 ip address 172.20.1.129 255.255.255.128
 duplex auto
 speed auto
 no shutdown
!
interface Serial0/0/0
 ip address 172.20.1.1 255.255.255.128
 clock rate 64000
 no shutdown
!
ip route 0.0.0.0 0.0.0.0 172.20.0.129
!
!
!
!
line con 0
line vty 0 4
password cisco
 login
!
end

Step 2 Load the following script onto the HQ router

hostname HQ
!
!
no ip domain-lookup
!
interface FastEthernet0/0
 ip address 172.20.0.129 255.255.255.128
 duplex auto
 speed auto
 no shutdown
!
interface Serial0/0/0
 ip address 172.20.1.2 255.255.255.128
 no shutdown
!
interface Serial0/0/1
 ip address 192.168.38.254 255.255.255.252
  no shutdown
!
ip route 192.168.39.64 255.255.255.192 192.168.38.253
!
!
!
!
line con 0
line vty 0 4
password cisco
 login
!
end

Step 3 Load the following script onto the ISP router

hostname ISP
!
!
no ip domain-lookup
!
interface FastEthernet0/0
 ip address 192.168.39.65 255.255.255.192
 duplex auto
 speed auto
!
interface Serial0/0/0
 ip address 192.168.38.253 255.255.255.252
 no shutdown
!
ip route 172.20.0.0 255.255.255.0 192.168.38.254
!
!
!
!
line con 0
line vty 0 4
password cisco
 login
!
end

Task 3: Troubleshoot the Branch Router

Step 1 Begin troubleshooting at the Host connected to the Branch router.

From the host PC1, is it possible to ping PC2? ___No________

From the host PC1, is it possible to ping the web server on the ISP LAN? _____No______

From the host PC1, is it possible to ping the default gateway? ___Yes________

Step 2 Next, examine the Branch router to find possible configuration errors. Begin by viewing the summary of status information for each interface on the router.

Are there any problems with the status of the interfaces? _No_

If there are any problems with the status of the interfaces, record any commands that will be necessary to correct the configuration errors. _None_

Step 3 If you have recorded any commands above, apply them to the router configuration now.

Step 4 If any changes were made to the configuration in the previous step, view the summary of the status information for the router interfaces again.

Does the information in the interface status summary indicate any configuration errors? ___No___
If the answer is yes, troubleshoot the interface status of the interfaces again.

Step 5 Troubleshoot the static route configuration on the Branch router. Begin by viewing the routing table.

What networks are shown in the routing table?

Directly connected networks (172.20.1.0/25 and 172.20.1.128/25) and a default route.

(Các mạng kết nối trực tiếp (172.20.1.0/25 và 172.20.1.128/25) và một default route)

Are there any problems with the routing table?

Yes. The default route points to the wrong next-hop IP (172.20.0.129). It should point to HQ's Serial0/0/0 IP (172.20.1.2).

(Có. Default route trỏ sai IP trạm kế tiếp (172.20.0.129). Nó phải trỏ về IP của cổng Serial0/0/0 trên HQ (172.20.1.2).

If there are any problems with the routing table, record any commands that will be necessary to correct the configuration errors.

Branch(config)# no ip route 0.0.0.0 0.0.0.0 172.20.0.129

Branch(config)# ip route 0.0.0.0 0.0.0.0 172.20.1.2

Step 6 If you have recorded any commands above, apply them to the router configuration now.

Step 7 If any changes were made to the configuration in the previous step, view the routing table again.

Does the information in routing table indicate any configuration errors? _____________
If the answer is yes, troubleshoot the routing table again.

Step 8 Attempt to ping between the hosts again.

From the host PC1, is it possible to ping PC2? ___No________

From the host PC1, is it possible to ping the web server on the ISP LAN? ____No_______

From the host PC1, is it possible to ping the Serial 0/0/0 interface of HQ? ___Yes_______

Task 4: Troubleshoot the HQ Router

Step 1 Begin troubleshooting at the Host connected to the HQ router.

From the host PC2, is it possible to ping PC1? ______No_____

From the host PC2, is it possible to ping the web server on the ISP LAN? _____No______

From the host PC2, is it possible to ping the default gateway? _____Yes_____

Step 2 Next, examine the HQ router to find possible configuration errors. Begin by viewing the summary of status information for each interface on the router.

Are there any problems with the status of the interfaces? __No__

If there are any problems with the status of the interfaces, record any commands that will be necessary to correct the configuration errors. __None__

Step 3 If you have recorded any commands above, apply them to the router configuration now.

Step 4 If any changes were made to the configuration in the previous step, view the summary of the status information for the router interfaces again.

Does the information in the interface status summary indicate any configuration errors? ____No_______
If the answer is yes, troubleshoot the interface status of the interfaces again.

Step 5 Troubleshoot the static routing configuration on the HQ router. Begin by viewing the routing table.

What networks are shown in the routing table?

Directly connected networks and a static route to 192.168.39.64.

(Các mạng kết nối trực tiếp và một static route đến 192.168.39.64.)

Are there any problems with the routing table?

Yes. HQ is missing a static route back to the Branch LAN (172.20.1.128/25).

(Có. HQ đang thiếu static route trả về mạng LAN của Branch (172.20.1.128/25))

If there are any problems with the routing table, record any commands that will be necessary to correct the configuration errors.

HQ(config)# ip route 172.20.1.128 255.255.255.128 172.20.1.1

Step 6 If you have recorded any commands above, apply them to the router configuration now.

Step 7 If any changes were made to the configuration in the previous step, view routing table again.

Does the information in routing table indicate any configuration errors? __No________
If the answer is yes, troubleshoot the routing table again.

Step 8 Attempt to ping between the hosts again.

From the host PC2, is it possible to ping PC1? ______Yes_____

From the host PC2, is it possible to ping the Serial 0/0/1 interface of the ISP router? _____No______

From the host PC1, is it possible to ping the web server on the ISP LAN? _____No______

Task 5: Troubleshoot the ISP Router

Step 1 Begin troubleshooting at the Host connected to the ISP router.

From Simulation mode, is it possible to ping from the Web Server on the ISP LAN to PC1? ______No_____

From Simulation mode, is it possible to ping from the Web Server on the ISP LAN to PC2? ______No_____

From Simulation mode, is it possible to ping from the Web Server on the ISP LAN to the default gateway? _____No______

Step 2 Next, examine the ISP router to find possible configuration errors. Begin by viewing the summary of status information for each interface on the router.

Are there any problems with the status of the interfaces?

Yes. FastEthernet0/0 is shut down. The serial connection is incorrectly configured on Serial0/0/0 instead of Serial0/0/1.

(Có. Cổng FastEthernet0/0 đang bị tắt. Kết nối Serial bị cấu hình sai vào cổng Serial0/0/0 thay vì Serial0/0/1)

If there are any problems with the status of the interfaces, record any commands that will be necessary to correct the configuration errors.


ISP(config)# interface FastEthernet0/0

ISP(config-if)# no shutdown

ISP(config)# interface Serial0/0/0

ISP(config-if)# no ip address

ISP(config-if)# shutdown

ISP(config)# interface Serial0/0/1

ISP(config-if)# ip address 192.168.38.253 255.255.255.252

ISP(config-if)# no shutdown

Step 3 If you have recorded any commands above, apply them to the router configuration now.

Step 4 If any changes were made to the configuration in the previous step, view the summary of the status information for the router interfaces again.

Does the information in the interface status summary indicate any configuration errors? ______No_____
If the answer is yes, troubleshoot the interface status of the interfaces again.

Step 5 Troubleshoot the static routing configuration on the ISP router. Begin by viewing the routing table.

What networks are shown in the routing table?


Directly connected networks and a static route to 172.20.0.0.

(Các mạng kết nối trực tiếp và một static route đến 172.20.0.0)

Are there any problems with the routing table?

Yes. The subnet mask 255.255.255.0 does not include the Branch network (172.20.1.0). It needs a /23 subnet mask (255.255.254.0) to summarize both HQ and Branch LANs.

(Có. Subnet mask 255.255.255.0 không bao gồm mạng Branch (172.20.1.0). Cần dùng subnet mask /23 (255.255.254.0) để gom cả mạng LAN của HQ và Branch.)

If there are any problems with the routing table, record any commands that will be necessary to correct the configuration errors.

ISP(config)# no ip route 172.20.0.0 255.255.255.0 192.168.38.254

ISP(config)# ip route 172.20.0.0 255.255.254.0 192.168.38.254

Step 6 If you have recorded any commands above, apply them to the router configuration now.

Step 7 if any changes were made to the configuration in the previous step, view routing table again.

Does the information in routing table indicate any configuration errors? _____No______
If the answer is yes, troubleshoot the routing table again.

Step 8 Attempt to ping between the hosts again.

From Simulation mode, is it possible to ping from the Web Server on the ISP LAN to PC1? _____Yes_____

From Simulation mode, is it possible to ping from the Web Server on the ISP LAN to PC2? _____Yes_____

From Simulation mode, is it possible to ping from the Web Server on the ISP LAN to the WAN interface of the Branch router? _____Yes_____

Task 6: Reflection

There were a number of configuration errors in the scripts that were provided for this lab. Use the space below to write a brief description of the errors that you found.

  Branch Router: The default static route was configured with the incorrect next-hop IP address.

  HQ Router: Missing a static route back to the Branch router's LAN network.

  ISP Router: The FastEthernet0/0 interface was not enabled (shutdown). The serial IP was assigned to the wrong interface (Serial0/0/0 instead of Serial0/0/1). The static route used an incorrect subnet mask, isolating the Branch network

(Branch Router: Default route được cấu hình sai địa chỉ IP trạm kế tiếp.)

(HQ Router: Thiếu static route trả về mạng LAN của Branch.)

(ISP Router: Cổng FastEthernet0/0 chưa được bật (bị shutdown). IP kết nối Serial bị gán nhầm cổng (Serial0/0/0 thay vì Serial0/0/1). Static route dùng sai subnet mask khiến mạng Branch bị cô lập.)