## 3.2.5 Convergence

### Learning Objectives:

- View the routing table of a converged network.

- Add a new LAN to the topology.

- View the debugging output.

- Watch the process of the network converging.

### Scenario

In this lab activity, the network has already been configured with 2 routers, 2 switches and 2 hosts. A new LAN will be added and you will watch the network converge.

### Task 1: View the routing tables in this network topology.

#### Step 1 View the routing table for Router2.

Router2#show ip route
Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP
D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area
N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2
E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP
i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area
* - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route

Gateway of last resort is not set

C 10.0.0.0/8 is directly connected, FastEthernet0/0
R 11.0.0.0/8 [120/1] via 192.168.1.2, 00:00:16, Serial0/0/0
C 192.168.1.0/24 is directly connected, Serial0/0/0
Router2#

#### Step 2 View the routing table for Router3.

Router3#show ip route
Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP
D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area
N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2
E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP
i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area
* - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route

Gateway of last resort is not set

R 10.0.0.0/8 [120/1] via 192.168.1.1, 00:00:16, Serial0/0/0
C 11.0.0.0/8 is directly connected, FastEthernet0/0
C 192.168.1.0/24 is directly connected, Serial0/0/0
Router3#

#### Step 3 Document the networks learned from RIP.
What network was learned via RIP updates for Router2?
Router2 learned the 11.0.0.0/8 network via the 192.168.1.2 interface

#### (Router2 đã học được mạng 11.0.0.0/8 thông qua cổng (interface) 192.168.1.2.)

What network was learned via RIP updates for Router3?
Router3 learned the 10.0.0.0/8 network via the 192.168.1.1 interface

(Router3 đã học được mạng 10.0.0.0/8 thông qua cổng 192.168.1.1.)

### Task 2: Add a new LAN to the network topology.

#### Step 1 Add a new 2960 switch and host to Router3.

#### Step 2 Cable the switch Fa0/1 interface to Router3’s Fa0/1 interface.

#### Step 3 Cable the host to the fa0/2 interface on the switch.

### Task 3: Configure the appropriate IP addresses from the 12.0.0.0/8 addressing scheme for this LAN.

#### Step 1 Use the first useable IP address on this network for the Fa0/1 interface on Router3.

Router3(config)#int fa0/1
Router3(config-if)#ip address 12.0.0.1 255.0.0.0
Do not enable the interface

#### Step 2 Configure the host with the last useable host address.

IP address 12.255.255.254
Subnet Mask 255.0.0.0
Default Gateway 12.0.0.1

#### Step 3 View the routing table for Router3 to see if this network has been added as a directly connected network.

Router3#show ip route
Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP
D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area
N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2
E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP
i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area
* - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route

Gateway of last resort is not set

R 10.0.0.0/8 [120/1] via 192.168.1.1, 00:00:16, Serial0/0/0
C 11.0.0.0/8 is directly connected, FastEthernet0/0
C 192.168.1.0/24 is directly connected, Serial0/0/0
Router3#

Why has this network not been added?
Because the FastEthernet0/1 interface on Router3 has not been enabled yet; it is still in an administratively down state.
(Vì cổng FastEthernet0/1 trên Router3 chưa được bật; nó vẫn đang ở trạng thái bị tắt bởi quản trị viên.)

#### Step 4 Ping from the LAN 3 host to the LAN 1 host.

Was the ping successful? No

### Task 4: Enable the Fa0/1 interface on Router3.

#### Step 1 Configure the no shutdown command on interface Fa0/1.

Router3(config-if)#no shutdown
%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up
%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

#### Step 2 View the routing table on Router3 to see if the newly added network appears.

Router3#show ip route
Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP
D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area
N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2
E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP
i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area
* - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route

Gateway of last resort is not set

R 10.0.0.0/8 [120/1] via 192.168.1.1, 00:00:19, Serial0/0/0
C 11.0.0.0/8 is directly connected, FastEthernet0/0
C 12.0.0.0/8 is directly connected, FastEthernet0/1
C 192.168.1.0/24 is directly connected, Serial0/0/0
Router3#

Has the network 12.0.0.0 been added to the Router3 routing table? Yes

### Task 5: Determine if the network has converged.

#### Step 1 View the routing table for Router2.

Router2#show ip route
Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP
D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area
N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2
E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP
i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area
* - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route

Gateway of last resort is not set

C 10.0.0.0/8 is directly connected, FastEthernet0/0
R 11.0.0.0/8 [120/1] via 192.168.1.2, 00:00:01, Serial0/0/0
C 192.168.1.0/24 is directly connected, Serial0/0/0
Router2#

Has the newly added network been added to the Router2 routing table? No
Why?
Although Router3 is physically connected to the new network, the RIP routing protocol on Router3 has not yet been configured to advertise the 12.0.0.0 network to its neighbors
(Mặc dù Router3 đã kết nối vật lý với mạng mới, nhưng giao thức định tuyến RIP trên Router3 chưa được cấu hình để quảng bá (advertise) mạng 12.0.0.0 cho các router láng giềng)

### Task 6: Turn on debugging on both routers.

Open the CLI session on both routers to observe output on both screens.

#### Step 1 Turn on debugging on Router2.

Router2#debug ip rip
RIP protocol debugging is on
Router2#

#### Step 2 Turn on debugging on Router3.

Router3#debug ip rip
RIP protocol debugging is on
Router3#

#### Step 3 Observe the debugging output on both routers.

Router3
Router3#RIP: sending v1 update to 255.255.255.255 via FastEthernet0/0 (11.0.0.1)
RIP: build update entries
network 10.0.0.0 metric 2
network 192.168.1.0 metric 1
RIP: sending v1 update to 255.255.255.255 via Serial0/0/0 (192.168.1.2)
RIP: build update entries
network 11.0.0.0 metric 1
Router2
Router2#RIP: received v1 update from 192.168.1.2 on Serial0/0/0
11.0.0.0 in 1 hops

Has Router3 included the new network 12.0.0.0 in the updates to Router2? No
Why?
As seen in the debug output, Router3 is only building update entries for the 10.0.0.0, 11.0.0.0, and 192.168.1.0 networks because those are the only networks currently included under the router rip configuration.
(Như được thấy trong kết quả debug, Router3 chỉ đang tạo các mục cập nhật cho các mạng 10.0.0.0, 11.0.0.0, và 192.168.1.0 bởi vì đó là những mạng duy nhất hiện đang nằm trong cấu hình router rip.)

### Task 7: Advertise the newly added network.

#### Step 1 Advertise network 12.0.0.0 with router rip on Router3.

Router3(config)#router rip
Router3(config-router)#network 12.0.0.0

#### Step 2 Observe the debugging output for both routers.

Router3

RIP: sending v1 request to 255.255.255.255 via FastEthernet0/1 (12.0.0.1)
RIP: sending v1 update to 255.255.255.255 via FastEthernet0/0 (11.0.0.1)
RIP: build update entries

network 10.0.0.0 metric 2
network 12.0.0.0 metric 1
network 192.168.1.0 metric 1

RIP: sending v1 update to 255.255.255.255 via Serial0/0/0 (192.168.1.2)

RIP: build update entries

network 11.0.0.0 metric 1
network 12.0.0.0 metric 1

RIP: sending v1 update to 255.255.255.255 via FastEthernet0/1 (12.0.0.1)

RIP: build update entries

network 10.0.0.0 metric 2
network 11.0.0.0 metric 1
network 192.168.1.0 metric 1

Router3(config-router)#

Router2

RIP: received v1 update from 192.168.1.2 on Serial0/0/0

11.0.0.0 in 1 hops
12.0.0.0 in 1 hops

RIP: received v1 update from 192.168.1.2 on Serial0/0/0

11.0.0.0 in 1 hops
12.0.0.0 in 1 hops

RIP: sending v1 update to 255.255.255.255 via FastEthernet0/0 (10.0.0.1)

RIP: build update entries

network 11.0.0.0 metric 2
network 12.0.0.0 metric 2
network 192.168.1.0 metric 1

RIP: sending v1 update to 255.255.255.255 via Serial0/0/0 (192.168.1.1)
RIP: build update entries

network 10.0.0.0 metric 1

Has a new network been added to the updates being sent to Router2? _Yes________

#### Step 3 Ping from the LAN 3 host to the LAN 1 host.

Was the ping successful? Yes

### Task 8: Turn debugging off on both routers.

#### Step 1 Turn debugging off on Router2.

Router2#no debug ip rip
RIP protocol debugging is off
Router2#

#### Step 2 Turn debugging off on Router3.

Router3#no debug ip rip
RIP protocol debugging is off
Router3#

### Task 9: Determine if the network has converged.

#### Step 1 View the routing table for Router2.

Router2#show ip route
Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP
D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area
N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2
E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP
i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area
* - candidate default, U - per-user static route, o - ODR P - periodic downloaded static route

Gateway of last resort is not set

C 10.0.0.0/8 is directly connected, FastEthernet0/0
R 11.0.0.0/8 [120/1] via 192.168.1.2, 00:00:01, Serial0/0/0
R 12.0.0.0/8 [120/1] via 192.168.1.2, 00:00:01, Serial0/0/0
C 192.168.1.0/24 is directly connected, Serial0/0/0
Router2#

Has the new network been added to the Router2 routing table? Yes

### Task 10: Reflection
What steps needed to be taken in order for the newly added network to converge?
First, the physical interface connected to the new LAN had to be configured with a valid IP address and enabled by using the no shutdown command. Second, the routing protocol (RIP) on the connected router (Router3) had to be updated with the network 12.0.0.0 command so it would actively advertise the existence of the new network to its neighbor, Router2.

(Đầu tiên, cổng vật lý kết nối với mạng LAN mới phải được cấu hình một địa chỉ IP hợp lệ và được bật lên bằng lệnh no shutdown. Thứ hai, giao thức định tuyến (RIP) trên router kết nối trực tiếp (Router3) phải được cập nhật bằng lệnh network 12.0.0.0 để nó chủ động quảng bá sự tồn tại của mạng mới này cho láng giềng của nó là Router2.)