## Lab 3.5.3: Subnetting Scenario 2

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| ISP | S0/0 | 209.165.200.227 | 255.255.255.224 | N/A |
| HQ | Fa0/0 |  |  | N/A |
|  | S0/0 |  |  | N/A |
|  | S0/1 |  |  | N/A |
|  | S0/2 | 209.165.200.226 | 255.255.255.224 | N/A |
| West | Fa0/0 |  |  | N/A |
|  | S0/0 |  |  | N/A |
|  | S0/1 |  |  | N/A |
|  | S0/2 |  |  | N/A |
| East | Fa0/0 |  |  | N/A |
|  | S0/0 |  |  | N/A |
|  | S0/1 |  |  | N/A |
|  | S0/2 |  |  | N/A |
| Branch 1 | Fa0/0 |  |  | N/A |
|  | S0/0 |  |  | N/A |
|  | S0/1 |  |  | N/A |
| Branch 2 | Fa0/0 |  |  | N/A |
|  | S0/0 |  |  | N/A |
|  | S0/1 |  |  | N/A |
| Branch 3 | Fa0/0 |  |  | N/A |
|  | S0/0 |  |  | N/A |
|  | S0/1 |  |  | N/A |
| Branch 4 | Fa0/0 |  |  | N/A |
|  | S0/0 |  |  | N/A |
|  | S0/1 |  |  | N/A |
| PC1 | NIC |  |  |  |
| PC2 | NIC |  |  |  |
| PC3 | NIC |  |  |  |
| PC4 | NIC |  |  |  |
| PC5 | NIC |  |  |  |
| PC6 | NIC |  |  |  |
| PC7 | NIC |  |  |  |

### Learning Objectives

- Determine the number of subnets needed.

- Determine the number of hosts needed.

- Design an appropriate addressing scheme.

- Assign addresses and subnet mask pairs to device interfaces and hosts.

- Examine the use of the available network address space.

- Determine how static routing could be applied to the network.

### Scenario

In this lab, you have been given the network address 172.16.0.0/16 to subnet and provide the IP addressing for the network shown in the Topology Diagram. The network has the following addressing requirements:

- The Branch 1 LAN will require 100 host IP addresses.

- The Branch 2 LAN will require 100 host IP addresses.

- The Branch 3 LAN will require 100 host IP addresses.

- The Branch 4 LAN will require 100 host IP addresses.

- The West LAN will require 400 hosts.

- The East LAN will require 400 hosts.

- The HQ LAN will require 500 host IP addresses.

- The links between each of the routers will require an IP address for each end of the link.

(Note: Remember that the interfaces of network devices are also host IP addresses and are included in the above addressing requirements.)

The IP addresses for the link from the HQ router to the ISP have already been assigned. The Serial 0/2 address of the HQ router is 209.165.200.226/27. The IP address of the Serial 0/0 of the ISP router is 209.165.200.227/27.

### Task 1: Examine the Network Requirements.

Examine the network requirements and answer the questions below. Keep in mind that IP addresses will be needed for each of the LAN interfaces.
How many subnets are needed? ____15_____

What is the maximum number of IP addresses that are needed for a single subnet? ____500____

How many IP addresses are needed for each of the branch LANs? ___100____

How many IP addresses are needed for all of the connections between routers? ____16___

What is the total number of IP addresses that are needed? ___1716 ___

### Task 2: Design an IP Addressing Scheme.

#### Step 1 Subnet the 172.16.0.0 network into the appropriate number of subnets. (Remember to allow for the maximum number of hosts required for the largest subnet.)

What will the subnet mask be for the subnetworks? __255.255.254.0___


How many usable host IP addresses are there per subnet? ___510 ___


Fill in the following chart with the subnet information.

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 172.16.0.0 |  |  |  |
| 1 | 172.16.2.0 | 172.16.2.1 | 172.16.2.254 | 172.16.3.255 |
| 2 | 172.16.4.0 | 172.16.4.1 | 172.16.4.254 | 172.16.5.255 |
| 3 | 172.16.6.0 | 172.16.6.1 | 172.16.6.254 | 172.16.7.255 |
| 4 | 172.16.8.0 | 172.16.8.1 | 172.16.8.254 | 172.16.9.255 |
| 5 | 172.16.10.0 | 172.16.10.1 | 172.16.10.254 | 172.16.11.255 |
| 6 | 172.16.12.0 | 172.16.12.1 | 172.16.12.254 | 172.16.13.255 |
| 7 | 172.16.14.0 | 172.16.14.1 | 172.16.14.254 | 172.16.15.255 |
| 8 | 172.16.16.0 | 172.16.16.1 | 172.16.16.254 | 172.16.17.255 |
| 9 | 172.16.18.0 | 172.16.18.1 | 172.16.18.254 | 172.16.19.255 |
| 10 | 172.16.20.0 | 172.16.20.1 | 172.16.20.254 | 172.16.21.255 |
| 11 | 172.16.22.0 | 172.16.22.1 | 172.16.22.254 | 172.16.23.255 |
| 12 | 172.16.24.0 | 172.16.24.1 | 172.16.24.254 | 172.16.25.255 |
| 13 | 172.16.26.0 | 172.16.26.1 | 172.16.26.254 | 172.16.27.255 |
| 14 | 172.16.28.0 | 172.16.28.1 | 172.16.28.254 | 172.16.29.255 |
| 15 | 172.16.30.0 | 172.16.30.1 | 172.16.30.254 | 172.16.31.255 |

#### Step 2 Assign the subnets to the network shown in the Topology Diagram.

When assigning the subnets, keep in mind that routing will need to occur to allow information to be sent throughout the network. The subnets will be assigned to the networks to allow for route summarization on each of the routers.

Note: The lowest subnet (subnet 0) will not be assigned in this lab. You should start assigning with the second lowest subnet (subnet 1).

- Assign subnet 1 to the Branch 1 LAN subnet. 172.16.2.0

- Assign subnet 2 to the Branch 2 LAN subnet. 172.16.4.0

- Assign subnet 3 to the link between the Branch 1 and Branch 2 routers. 172.16.6.0

- Assign subnet 4 to the link between the Branch 1 and West routers. 172.16.8.0

- Assign subnet 5 to the link between the Branch 2 and West routers. 172.16.10.0

- Assign subnet 6 to the West LAN subnet. 172.16.12.0

- Assign subnet 7 the link between the West and HQ routers. 172.16.14.0

- Assign subnet 8 to the HQ LAN subnet. 172.16.16.0

- Assign subnet 9 to the link between the HQ and East routers. 172.16.18.0

- Assign subnet 10 to the East LAN subnet. 172.16.20.0

- Assign subnet 11 to the link between the Branch 3 and East routers. 172.16.22.0

- Assign subnet 12 to the link between the Branch 4 and East routers. 172.16.24.0

- Assign subnet 13 to the link between the Branch 3 and Branch 4 routers. 172.16.26.0

- Assign subnet 14 to the Branch 3 LAN subnet. 172.16.28.0

- Assign subnet 15 to the Branch 4 LAN subnet. 172.16.30.0

### Task 3: Assign IP Addresses to the Network Devices.

Assign the appropriate addresses to the device interfaces. Document the addresses to be used in the table provided under the Topology Diagram.

#### Step 1 Assign addresses to the HQ router.
• Assign the first valid host address in the HQ LAN subnet to the LAN interface.
• Assign the first valid host address in the link between the HQ and West routers subnet to the S0/0 interface.
• Assign the first valid host address in the link between the HQ and East routers subnet to the S0/1 interface.

#### Step 2 Assign addresses to the West router.
• Assign the first valid host address in the West LAN subnet to the LAN interface.
• Assign the last valid host address in the link between the HQ and West routers subnet to the S0/0 interface.
• Assign the first valid host address in the link between the West and Branch 1 routers subnet to the S0/1 interface.
• Assign the first valid host address in the link between the West and Branch 2 routers subnet to the S0/2 interface.

#### Step 3 Assign addresses to the East router.
• Assign the first valid host address in the East LAN subnet to the LAN interface.
• Assign the last valid host address in the link between the HQ and East routers subnet to the S0/0 interface.
• Assign the first valid host address in the link between the East and Branch 3 routers subnet to the S0/1 interface.
• Assign the first valid host address in the link between the East and Branch 4 routers subnet to the S0/2 interface.

#### Step 4 Assign addresses to the Branch 1 router.
• Assign the first valid host address in the Branch 1 LAN subnet to the LAN interface.
• Assign the last valid host address in the link between the West and Branch 1 routers subnet to the S0/0 interface.
• Assign the first valid host address in the link between the Branch 1 and Branch 2 routers subnet to the S0/1 interface.

#### Step 5 Assign addresses to the Branch 2 router.
• Assign the first valid host address in the Branch 2 LAN subnet to the LAN interface.
• Assign the last valid host address in the link between the West and Branch 2 routers subnet to the S0/0 interface.
• Assign the last valid host address in the link between the Branch 1 and Branch 2 routers subnet to the S0/1 interface.

#### Step 6 Assign addresses to the Branch 3 router.
• Assign the first valid host address in the Branch 3 LAN subnet to the LAN interface.
• Assign the last valid host address in the link between the East and Branch 3 routers subnet to the S0/0 interface.
• Assign the first valid host address in the link between the Branch 3 and Branch 4 routers subnet to the S0/1 interface.

#### Step 7 Assign addresses to the Branch 4 router.
• Assign the first valid host address in the Branch 4 LAN subnet to the LAN interface.
• Assign the last valid host address in the link between the East and Branch 4 routers subnet to the S0/0 interface.
• Assign the last valid host address in the link between the Branch 3 and Branch 4 routers subnet to the S0/1 interface.

#### Step 8 Assign addresses to the host PCs.
• Assign the last valid host address in the HQ LAN subnet PC1.
• Assign the last valid host address in the West LAN subnet to PC2.
• Assign the last valid host address in the East 1 LAN subnet to PC3.
• Assign the last valid host address in the Branch 1 LAN subnet to PC4.
• Assign the last valid host address in the Branch 2 LAN subnet to PC5.
• Assign the last valid host address in the Branch 3 LAN subnet to PC6.
• Assign the last valid host address in the Branch 4 LAN subnet to PC7.

### Task 4: Test the Network Design.

Apply your addressing scheme to the Packet Tracer file that has been supplied with this lab. Check to see that all devices on directly connected networks can ping each other.

### Task 5: Reflection

How many IP address in the 172.16.0.0 network are wasted in this design?

A large number of IP addresses are wasted. The 172.16.0.0/16 network provides 65,536 addresses. By using 15 subnets of /23 (512 addresses per block), we only allocate 7,680 IPs. Furthermore, WAN links only need 2 IPs but are assigned 510 usable IPs, wasting 508 IPs per link.

(Rất nhiều IP bị lãng phí. Mạng 172.16.0.0/16 cung cấp tổng cộng 65.536 địa chỉ IP. Trong thiết kế này, chúng ta sử dụng 15 subnets, mỗi subnet tốn một block 512 địa chỉ, tổng cộng là 7.680 IP. Hơn nữa, vì dùng chia mạng cố định FLSM (/23), các đường link WAN chỉ cần 2 IP nhưng lại được cấp tới 510 IP khả dụng, gây lãng phí 508 IP cho mỗi kết nối.)

What would the command be to add a default static route for your entire network design from the HQ router to the ISP router? ip route 0.0.0.0 0.0.0.0 209.165.200.227

Can the West, Branch 1, and Branch 2 networks be summarized into one route on the HQ router? Yes

This summarized route should also include the serial links connected the West, Branch 1, and Branch 2 routers. What would be the command used to add this summary route to the routing table? ip route 172.16.0.0 255.255.240.0 Serial0/0

Can the East, Branch 3, and Branch 4 networks be summarized into one route on the HQ router? Yes

This summarized route should also include the serial links that connect the East, Branch 3, and Branch 4 routers.  What would be the command used to add this summary route to the routing table? ip route 172.16.16.0 255.255.240.0 Serial0/1

What would the command be to add a default static route on the West router to send traffic for all unknown destinations to the HQ router? ip route 0.0.0.0 0.0.0.0 Serial0/0

What would the command be to add a default static route on the East router to send traffic for all unknown destinations to the HQ router? ip route 0.0.0.0 0.0.0.0 Serial0/0

Can the Branch 1 and Branch 2 networks be summarized into one route on the West router? Yes

This summarized route should also include the serial link that connects the Branch 1 and Branch 2 routers. What would be the command used to add this summary route to the routing table? Use the s0/1 interface of the West router as the exit interface.
ip route 172.16.0.0 255.255.248.0 Serial0/1

Can the Branch 3 and Branch 4 networks be summarized into one route on the East router? Yes

This summarized route should also include the serial link that connects the Branch 3 and Branch 4 routers. What would be the command used to add this summary route to the routing table? Use the s0/1 interface of the East router as the exit interface.
ip route 172.16.24.0 255.255.248.0 Serial0/1

The Branch 1 router requires a static route for traffic destined for Branch 2. All other traffic should be sent to the West router using a default static route. What commands would be used to accomplish this?

ip route 172.16.4.0 255.255.254.0 Serial0/1

ip route 0.0.0.0 0.0.0.0 Serial0/0

The Branch 2 router requires a static route for traffic destined for Branch 1. All other traffic should be sent to the West router using a default static route. What commands would be used to accomplish this?

ip route 172.16.2.0 255.255.254.0 Serial0/1

ip route 0.0.0.0 0.0.0.0 Serial0/0

The Branch 3 router requires a static route for traffic destined for Branch 4. All other traffic should be sent to the East router using a default static route. What commands would be used to accomplish this?

ip route 172.16.30.0 255.255.254.0 Serial0/1

ip route 0.0.0.0 0.0.0.0 Serial0/0

The Branch 4 router requires a static route for traffic destined for Branch 3. All other traffic should be sent to the East router using a default static route. What commands would be used to accomplish this?

ip route 172.16.28.0 255.255.254.0 Serial0/1

ip route 0.0.0.0 0.0.0.0 Serial0/0