Lab 11.6.2: Challenge OSPF Configuration Lab

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| HQ | Fa0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 |  |  | N/A |
|  | Lo1 |  |  | N/A |
| Branch1 | Fa0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 |  |  | N/A |
| Branch2 | Fa0/0 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
|  | S0/0/1 |  |  | N/A |
| PC1 | NIC |  |  |  |
| PC2 | NIC |  |  |  |
| PC3 | NIC |  |  |  |

Learning Objectives

Upon completion of this lab, you will be able to:

- Create an efficient VLSM design given requirements

- Assign appropriate addresses to interfaces and document

- Cable a network according to the Topology Diagram

- Erase the startup configuration and reload a router to the default state

- Configure routers including OSPF

- Configure and propagate a static default route

- Verify OSPF operation

- Test and verify full connectivity

- Reflect upon and document the network implementation

Scenario

In this lab activity, you will be given a network address that must be subnetted using VLSM to complete the addressing of the network shown in the Topology Diagram. A combination OSPF routing and static routing will be required so that hosts on networks that are not directly connected will be able to communicate with each other. OSPF area ID of 0 and process ID of 1 will be used in all OSPF configurations.

Task 1: Subnet the Address Space.

Step 1: Examine the network requirements.

The addressing for the Network has the following requirements.

- The 172.20.0.0/16 network must be subnetted to provide addresses for the LANs and serial links.

  - The HQ LAN will require 8000 addresses

  - The Branch1 LAN will require 4000 addresses

  - The Branch 2 LAN will require 2000 addresses

  - The links between the routers will require two addresses for each link

- The loopback address representing the link between the HQ router and the ISP will use the 10.10.10.0/30 network.

Step 2: Consider the following questions when creating your network design.

How many subnets need to be created from the 172.20.0.0/16 network? ___6____

How many total IP addresses are required from the 172.16.0.0/16 network? ___14,006__

What subnet mask will be used for the HQ LAN subnet? ____255.255.224.0________

What is the maximum number of host addresses that could be used on this subnet?_8190_

What subnet mask will be used for the Branch1 LAN subnet? ____255.255.240.0______

What is the maximum number of host addresses that could be used on this subnet?_4094_

What subnet mask will be used for the Branch2 LAN subnet? ____255.255.248.0____

What is the maximum number of host addresses that could be used on this subnet?_2046_

What subnet mask will be used for the links between the three routers? _____255.255.255.252______

What is the maximum number of host addresses that could be used on each of these subnets? ___2____

Step 3: Assign subnetwork addresses to the Topology Diagram.

- Assign subnet 0 of the 172.20.0.0/16 network to the HQ LAN subnet. What is the network address of this subnet? ____172.20.0.0____

- Assign subnet 1 of the 172.20.0.0/16 network to the Branch1 LAN subnet. What is the network address of this subnet? ______172.20.32.0______

- Assign subnet 2 of the 172.20.0.0/16 network to the Branch2 LAN subnet. What is the network address of this subnet? ______172.20.48.0_____

- Assign subnet 3 of the 172.20.0.0/16 network to the link between the HQ and Branch1 routers. What is the network address of this subnet? _______172.20.56.0________

- Assign subnet 4 of the 172.20.0.0/16 network to the link between the HQ and Branch2 routers. What is the network address of this subnet? ________172.20.56.4________

- Assign subnet 5 of the 172.20.0.0/16 network to the link between the Branch1 and Branch2 routers. What is the network address of this subnet? _____172.20.56.8______

Task 2: Determine Interface Addresses.

Assign appropriate addresses to the device interfaces.

- Assign the first valid host address in the 10.10.10.0/30 network to the Loopback 1 interface on the HQ router.

- Assign the first valid IP address of the HQ LAN network to the LAN interface of the HQ router.

- Assign the last valid IP address of the HQ LAN network to PC2.

- Assign the first valid IP address of the Branch1 LAN network to the LAN interface of the Branch1 router.

- Assign the last valid IP address of the Branch1 LAN network to PC1.

- Assign the first valid IP address of the Branch2 LAN network to the LAN interface of the Branch2 router.

- Assign the last valid IP address of the Branch2 LAN network to PC3.

- Assign the first valid IP address of the HQ to Branch1 link network to the Serial 0/0/0 interface of the HQ router.

- Assign the last valid IP address of the HQ to Branch1 link network to the Serial0/0/0 interface of the Branch router.

- Assign the first valid IP address of the HQ to Branch2 link network to the Serial 0/0/1 interface of the HQ router.

- Assign the last valid IP address of the HQ to Branch2 link network to the Serial0/0/1 interface of the Branch2 router.

- Assign the first valid IP address of the Branch1 to Branch2 link network to the Serial 0/0/1 interface of the Branch1 router.

- Assign the last valid IP address of the Branch1 to Branch2 link network to the Serial0/0/0 interface of the Branch2 router.

Document the addresses to be used in the table provided under the Topology Diagram.

Task 3: Prepare the Network.

Step 1: Cable a network that is similar to the one in the Topology Diagram.

You can use any current router in your lab as long as it has the required interfaces as shown in the topology.

Step 2: Clear any existing configurations on the routers.

Task 4: Perform Basic Router Configurations.

Perform basic configuration of the BRANCH, HQ, and ISP routers according to the following guidelines:

- Configure the router hostname.

- Disable DNS lookup.

- Configure an EXEC mode password.

- Configure a message-of-the-day banner.

- Configure a password for console connections.

- Configure a password for VTY connections.

- Synchronize unsolicited messages and debug output with solicited output and prompts for the console and virtual terminal lines.

- Configure an EXEC timeout of 15 minutes.

Task 5: Configure and Activate Serial and Ethernet Addresses.

Step 1: Configure the interfaces on the HQ, Branch1, and Branch2 routers with the IP addresses from the table provided under the Topology Diagram.

When you have finished, be sure to save the running configuration to the NVRAM of the router.

Step 2: Configure the Ethernet interfaces of PC1, PC2, and PC3 with the IP addresses from the table provided under the Topology Diagram. Configure serial DCE interfaces with clock rates of 64000.

Step 3: Configure the correct bandwidth for the serial interfaces on the Branch 1 router.

What commands are required to accomplish this? ____bandwidth 64____

Step 4: Configure the correct bandwidth for the serial interfaces on the Branch 2 router.

What commands are required to accomplish this?_____ bandwidth 64______________

Step 5: Configure the correct bandwidth for the serial interfaces on the HQ router.

What commands are required to accomplish this?_____ bandwidth 64____________

Task 6: Verify Connectivity to Next Hop Device.

You should NOT have connectivity between end devices yet. However, you can test connectivity between two routers and between and end device and its default gateway.

Step 1: Verify that the HQ, Branch1, and Branch2 routers can ping each of the neighboring routers across the WAN links.

Step 2: Verify that PC1, PC2, and PC3 can ping their respective default gateway.

Task 7: Configure OSPF Routing on the Branch1 Router.

Step 1: Consider the networks that need to be included in the OSPF updates that are sent out by the Branch1 router.

What directly connected networks are present in the Branch1 routing table?
_________ 172.20.32.0/20, 172.20.56.0/30, 172.20.56.8/30_________________

What commands are required to enable OSPF and include the connected networks in the routing updates?
router ospf 1

network 172.20.32.0 0.0.15.255 area 0

network 172.20.56.0 0.0.0.3 area 0

network 172.20.56.8 0.0.0.3 area 0

Are there any router interfaces that do not need to have OSPF updates sent out?__Yes___

What command is used to disable OSPF updates on these interfaces?
___________ passive-interface FastEthernet0/0_________________________

Task 8: Configure OSPF and Static Routing on the HQ Router.

Step 1: Consider the type of static routing that is needed on HQ.

A static default route will need to be configured to send all packets with destination addresses that are not in the routing table to the loopback address representing the link between the HQ router and the ISP. What command is needed to accomplish this?
__________ ip route 0.0.0.0 0.0.0.0 loopback 1____________________

What directly connected networks are present in the HQ routing table?
__________ 172.20.0.0/19, 172.20.56.0/30, 172.20.56.4/30, 10.10.10.0/30____________

Will the networks of the HQ LAN and the links between the Branch 1 and Branch2 routers need to have the subnet mask information included in the network statements? __Yes___

What commands are required to enable OSPF and include the appropriate networks in the routing updates?
router ospf 1

network 172.20.0.0 0.0.31.255 area 0

network 172.20.56.0 0.0.0.3 area 0

network 172.20.56.4 0.0.0.3 area 0

Are there any router interfaces that do not need to have OSPF updates sent out?__Yes___

What command is used to disable OSPF updates on these interfaces?
___ passive-interface FastEthernet0/0 and passive-interface loopback 1__________

The HQ router needs to send the default route information to the Branch1 and Branch2 routers in the OSPF updates. What command is used to configure this?
__________ default-information originate__________________

Task 9: Configure OSPF Routing on the Branch2 Router.

Step 1: Consider the networks that need to be included in the OSPF updates that are sent out by the Branch2 router.

What directly connected networks are present in the Branch2 routing table?
____________ 172.20.48.0/21, 172.20.56.4/30, 172.20.56.8/30___________________

What commands are required to enable OSPF and include the connected networks in the routing updates?
router ospf 1

network 172.20.48.0 0.0.7.255 area 0

network 172.20.56.4 0.0.0.3 area 0

network 172.20.56.8 0.0.0.3 area 0

Are there any router interfaces that do not need to have OSPF updates sent out? _Yes___

What command is used to disable OSPF updates on these interfaces?
________ passive-interface FastEthernet0/0______________________

Task 10: Verify the Configurations

Answer the following questions to verify that the network is operating as expected.

From PC1, is it possible to ping PC2? ____Yes______
From PC1, is it possible to ping the PC3? ___Yes______

The answer to the above questions should be ‘yes’. If any of the above pings failed, check your physical connections and configurations. Refer to your basic troubleshooting techniques used in the Chapter 1 labs.

What OSPF routes are present in the routing table of the Branch1 router?
172.20.0.0/19 (HQ LAN), 172.20.48.0/21 (Branch2 LAN), and 172.20.56.4/30 (Link HQ - Branch2).

What is the gateway of last resort in the routing table of the Branch1 router?
_________ 172.20.56.1______________________________

What OSPF routes are present in the routing table of the HQ router?
172.20.32.0/20 (Branch1 LAN), 172.20.48.0/21 (Branch2 LAN), và 172.20.56.8/30 (Link Branch1 - Branch2).

What is the gateway of last resort in the routing table of the HQ router?
_______ 0.0.0.0 to network 0.0.0.0 (via Loopback 1 / 10.10.10.1)__________

What OSPF routes are present in the routing table of the Branch2 router?
172.20.0.0/19 (HQ LAN), 172.20.32.0/20 (Branch1 LAN), và 172.20.56.0/30 (Link HQ - Branch1).

What is the gateway of last resort in the routing table of the Branch2 router?
_______________ 172.20.56.5 ________________________________

Task 11: Reflection

On PC1, use the tracert command to examine the route that is used between PC1 and PC3.
What are the hops in the route to PC3?
172.20.32.1 (Branch1) -> 172.20.56.1 (HQ) -> 172.20.56.6 (Branch2) -> 172.20.55.254 (PC3).

Is this the least number of hops that can be used to reach PC3? ___No________

If the answer is no, why is a path with more than the minimum amount of hops used?

Because OSPF is a link-state routing protocol, it uses cost (which is calculated based on bandwidth) to determine the best path. The direct link between Branch1 and Branch2 is a low-speed connection (64 kbps, resulting in a very high cost). In contrast, the links connecting to HQ are T1 lines (1.544 Mbps, resulting in a much lower cost).

Therefore, the total cost of routing the traffic through HQ is less than taking the direct route. As a result, OSPF prioritizes the indirect path with the higher bandwidth, even though it requires more hops. (This is a major advantage of OSPF over the RIP protocol).

(Bởi vì OSPF là giao thức định tuyến theo trạng thái liên kết (link-state) và nó sử dụng cost (được tính toán dựa trên băng thông - bandwidth) để chọn đường đi tốt nhất. Đường link trực tiếp giữa Branch1 và Branch2 là đường truyền tốc độ thấp (64 kbps, cost rất cao). Ngược lại, các đường link nối với HQ là đường T1 (1.544 Mbps, cost thấp hơn nhiều). Do đó, tổng cost của việc đi vòng qua HQ nhỏ hơn việc đi trực tiếp, nên OSPF đã ưu tiên chọn đường vòng có băng thông rộng hơn dù nó tốn nhiều hop hơn.)