Packet Tracer - Basic Router Configuration Review

# Addressing Table

| Device | Interface | IP Address / Prefix | Default Gateway |
| --- | --- | --- | --- |
| R2 | G0/0/0 | 10.0.4.1 /24 | N/A |
| R2 | G0/0/0 | 2001:db8:acad:4::1 /64 | N/A |
| R2 | G0/0/0 | fe80::2:a | N/A |
| R2 | G0/0/1 | 10.0.5.1 /24 | N/A |
| R2 | G0/0/1 | 2001:db8:acad:5::1 /64 | N/A |
| R2 | G0/0/1 | fe80::2:b | N/A |
| R2 | S0/1/0 | 10.0.3.2 /24 | N/A |
| R2 | S0/1/0 | 2001:db8:acad:3::2 /64 | N/A |
| R2 | S0/1/0 | fe80::1:c | N/A |
| R2 | S0/1/1 | 209.165.200.225 /30 | N/A |
| R2 | S0/1/1 | 2001:db8:feed:224::1/64 | N/A |
| R2 | S0/1/1 | fe80::1:d | N/A |
| PC1 | NIC | 10.0.1.10 /24 | 10.0.1.1 |
| PC1 | NIC | 2001:db8:acad:1::10 /64 | fe80::1:a |
| PC2 | NIC | 10.0.2.10 /24 | 10.0.2.1 |
| PC2 | NIC | 2001:db8:acad:2::10 /64 | fe80::1:b |
| PC3 | NIC | 10.0.4.10 /24 | 10.0.4.1 |
| PC-3 | NIC | 2001:db8:acad:4::10 /64 | fe80::2:a |
| PC4 | NIC | 10.0.5.10 /24 | 10.0.5.1 |
| PC-4 | NIC | 2001:db8:acad:5::10 /64 | fe80::2:b |

Blank Line - no additional information

# Objectives

Part 1: Configure Devices and Verify Connectivity

=   Assign static IPv4 and IPv6 addresses to the PC interfaces.

=   Configure basic router settings.

=   Configure the router for SSH.

=   Verify network connectivity.

Part 2: Display Router Information

=   Retrieve hardware and software information from the router.

=   Interpret the startup configuration.

=   Interpret the routing table.

=   Verify the status of the interfaces.

# Background / Scenario

This activity requires you to configure the R2 router using the settings from the Addressing Table and the specifications listed. The R1 router and the devices connected to it have been configured. This is a comprehensive review of previously covered IOS router commands. In Part 1, you will complete basic configurations and interface settings on the router. In Part 2, you will use SSH to connect to the router remotely and utilize the IOS commands to retrieve information from the device to answer questions about the router. For review purposes, this lab provides the commands necessary for specific router configurations.

# Instructions

## Part 1: Configure Devices and Verify Connectivity

### Step 1: Configure the PC interfaces.

a.     Configure the IPv4 and IPv6 addresses on PC3 as listed in the Addressing Table.

b.     Configure the IPv4 and IPv6 addresses on PC4 as listed in the Addressing Table.

### Step 2: Configure the router.

Open configuration window

a.     On the R2 router, open a terminal. Move to privileged EXEC mode.

b.     Enter configuration mode.

c.     Assign a device name of R2 to the router.

d.     Configure c1sco1234 as the encrypted privileged EXEC mode password.

e.     Set the domain name of the router to ccna-lab.com.

f.      Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

g.     Encrypt the plaintext passwords.

h.     Configure the username SSHadmin with an encrypted password of 55Hadm!n.

i.      Generate a set of crypto keys with a 1024 bit modulus.

j.      Assign cisco as the console password, configure sessions to disconnect after six minutes of inactivity, and enable login. To prevent console messages from interrupting commands, use the logging synchronous command.

k.     Assign cisco as the vty password, configure the vty lines to accept SSH connections only, configure sessions to disconnect after six minutes of inactivity, and enable login using the local database.

l.      Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

m.   Enable IPv6 Routing.

n.     Configure all four interfaces on the router with the IPv4 and IPv6 addressing information from the addressing table above. Configure all four interfaces with descriptions. Activate all four interfaces.

o.     Save the running configuration to the startup configuration file.

Close configuration wind

### Step 3: Verify network connectivity.

a.     Using the command line at PC3, ping the IPv4 and IPv6 addresses for PC4.estion:

Were the pings successful? Yes

b.     From the CLI on R2 ping the S0/1/1 address of R1 for both IPv4 and IPv6. The addresses assigned to the S0/1/1 interface on R1 are:

IPv4 address = 10.0.3.1

IPv6 address = 2001:db8:acad:3::1

Were the pings successful? Yes

From the command line of PC3 ping the ISP address 209.165.200.226.stion:

Were the pings successful? Yes

From PC3 attempt to ping an address on the ISP for testing, 64.100.1.1.tion:

Were the pings successful? No

c.     From the command line of PC3 open an SSH session to the R2 G0/0/0 IPv4 address and log in as SSHadmin with the password 55Hadm!n.

C:\> ssh -l SSHadmin 10.0.4.1

Password:

Was remote access successful? Yes

## Part 2: Display Router Information

In Part 2, you will use show commands from an SSH session to retrieve information from the router.

### Step 1: Establish an SSH session to R2.

From the command line of PC3 open an SSH session to the R2 G0/0/0 IPv6 address and log in as SSHadmin with the password 55Hadm!n.

### Step 2: Retrieve important hardware and software information.

a.     Use the show version command to answer questions about the router.

What is the name of the IOS image that the router is running?

System image file is "bootflash:/isr4300-universalk9.03.16.05.S.155-3.S5-ext.SPA.bin"

How much non-volatile random-access memory (NVRAM) does the router have?

32768K bytes of non-volatile configuration memory

How much Flash memory does the router have?

3223551K bytes of flash memory at bootflash

b.     The show commands often provide multiple screens of outputs. Filtering the output allows a user to display certain sections of the output. To enable the filtering command, enter a pipe (|) character after a show command, followed by a filtering parameter and a filtering expression. You can match the output to the filtering statement by using the include keyword to display all lines from the output that contain the filtering expression. Filter the show version command, using show version | include register to answer the following question.

What is the boot process for the router on the next reload?

Configuration register is 0x2102

### Step 3: Display the running configuration.

Use the show running-config command on the router to answer the following questions filtering for lines containing the word “password”.esti

How are passwords presented in the output?

Mật khẩu hiển thị ở dạng mã hóa (encrypted/hashed) thành các chuỗi ký tự khó đọc, thay vì là plaintext (nhờ lệnh service password-encryption và secret).

Use the show running-config | begin vty command.

What is the result of using this command?

Lệnh này giúp hiển thị nội dung file cấu hình đang chạy, nhưng không in ra từ đầu mà bắt đầu in từ dòng đầu tiên có chứa chữ "vty" cho đến hết file.

Note: A more specific command would be show running-config | section vty; however, the current version of Packet Tracer does not support the section filtering command.

### Step 4: Display the routing table on the router.

Use the show ip route command on the router to answer the following questions.

#### Question:

What code is used in the routing table to indicate a directly connected network?

C (Connected)

How many route entries are coded with a C code in the routing table? 4

### Step 5: Display a summary list of the interfaces on the router.

a.     Use the show ip interface brief command on the router to answer the following question.

#### Questions:

What command changed the status of the Gigabit Ethernet ports from administratively down to up? no shutdown

What filtering command would you use to display only the interfaces with addresses assigned? show ip interface brief | exclude unassigned

b.     Use the show ipv6 int brief command to verify IPv6 settings on R2.

#### Question:

What is the meaning of the [up/up] part of the output?

- Chữ up đầu tiên thể hiện trạng thái lớp vật lý (Physical Layer / Layer 1) - nghĩa là cáp đã cắm đúng và có tín hiệu kết nối.

- Chữ up thứ hai thể hiện trạng thái giao thức liên kết dữ liệu (Data Link Layer / Layer 2) - nghĩa là giao thức đường truyền hoạt động bình thường, các gói tin (keepalives) có thể giao tiếp thành công.

End of document