Packet Tracer - Troubleshoot Static and Default Routes

# Addressing Table

| Device | Interface | IP Addresses |
| --- | --- | --- |
| R1 | G0/0 | 172.31.1.1/25 |
| R1 | G0/0 | 2001:DB8:1::1/64 |
| R1 | S0/0/0 | 172.31.1.194/30 |
| R1 | S0/0/0 | 2001:DB8:2::194/64 |
| R2 | G0/0 | 172.31.0.1/24 |
| R2 | G0/0 | 2001:DB8:3::1/64 |
| R2 | S0/0/0 | 172.31.1.193/30 |
| R2 | S0/0/0 | 2001:DB8:2::193/64 |
| R2 | S0/0/1 | 172.31.1.197/30 |
| R2 | S0/0/1 | 2001:DB8:4::197/64 |
| R3 | G0/0 | 172.31.1.129/26 |
| R3 | G0/0 | 2001:DB8:5::1/64 |
| R3 | S0/0/1 | 172.31.1.198/30 |
| R3 | S0/0/1 | 2001:DB8:4::198/64 |
| PC1 | NIC | 172.31.1.126/25 |
| PC1 | NIC | 2001:DB8:1::126/64 |
| PC2 | NIC | 172.31.0.254/24 |
| PC2 | NIC | 2001:DB8:3::254/64 |
| Server | NIC | 172.31.1.190/26 |
| Server | NIC | 2001:DB8:5::190/64 |

Blank Line - no additional information

# Objectives

In this activity you will troubleshoot static and default routes and repair any errors that you find.

=   Troubleshoot IPv4 static routes.

=   Troubleshoot IPv4 default routes.

=   Troubleshoot IPv6 static routes.

=   Configure IPv4 static routes.

=   Configure IPv4 default routes.

=   Configure IPv6 static routes.

# Background / Scenario

A newly hired network technician is attempting to preconfigure a simple topology that will be delivered to a customer. The technician has not been able to establish connectivity between the three LANs. You have been asked to troubleshoot the topology and verify connectivity between the hosts on the three LANs over IPv4 and IPv6.

# Instructions

## Part 1: Locate and document the problems.

Record your findings in a table like the one below.

| Location | Problem | Solution |
| --- | --- | --- |
| R1 | Incorrect Next-hop address for the IPv4 default route (172.31.1.195 is the broadcast address, not R2's IP). Cấu hình sai địa chỉ Next-hop cho IPv4 Default Route (172.31.1.195 là địa chỉ broadcast, không phải IP của R2). | Remove the incorrect route and change the Next-hop to R2's correct S0/0/0 IP address, which is 172.31.1.193. Xóa route sai và đổi Next-hop về đúng IP cổng S0/0/0 của R2 là 172.31.1.193. |
| R2 | IPv4 Next-hop addresses for LAN 1 and LAN 3 are reversed/swapped. Cấu hình ngược hướng Next-hop IPv4 cho mạng LAN 1 và LAN 3. | Remove the two incorrect routes and swap the Next-hops: LAN 1 should point to 172.31.1.194, and LAN 3 should point to 172.31.1.198. Xóa 2 route sai và đổi chéo lại Next-hop: LAN 1 trỏ về 172.31.1.194, LAN 3 trỏ về 172.31.1.198. |
| R2 | The IPv6 static route to LAN 1 has a typo in the destination network address (DB6 instead of DB8). Tuyến đường IPv6 đi LAN 1 bị gõ sai địa chỉ mạng đích (gõ nhầm DB6 thay vì DB8). | Remove the incorrect route and reconfigure it with the correct destination network address: 2001:DB8:1::/64. Xóa route sai và cấu hình lại với địa chỉ mạng đích đúng là 2001:DB8:1::/64. |
| R3 | The IPv4 static route to LAN 1 is configured with the wrong Subnet Mask (uses 255.255.255.240 instead of the standard /25). Tuyến đường IPv6 đi LAN 1 bị gõ sai địa chỉ mạng đích (gõ nhầm DB6 thay vì DB8). | Remove the incorrect route and reconfigure it with the correct Subnet Mask: 255.255.255.128. Xóa route sai và cấu hình lại với Subnet Mask chuẩn là 255.255.255.128. |
| R3 | Missing an IPv6 static route to the LAN 1 network. Thiếu tuyến đường IPv6 (Static Route) để đi đến mạng LAN 1. | Add a new IPv6 static route pointing to the 2001:DB8:1::/64 network via the Serial0/0/1 interface. Bổ sung tuyến đường IPv6 tĩnh hướng đến mạng 2001:DB8:1::/64 đi qua cổng Serial0/0/1. |

Blank Line - no additional information

## Part 2: Repair the problems.

Configure the devices so that full connectivity exists between the hosts on the LANs over IPv4 and IPv6.

Note: Your task is to establish connectivity using the existing static route design. Changing the types of static routes used will result in a loss of points.

End of document